Smithfield Shopping Plaza; Printing Co Expansion

Existing Conditions by room number

Room number 1} Storefront door and glazing 9’0” x9’0”; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down carpet; plastic laminated reception counter and base, plastic laminated base and counter top; 2 x 4 Acoustical ceiling 8’9” AFF, painted walls; AC, Fire Sprinkler, Electrical 2 x 4 lights and standard electrical receptacles.

Room number 2} Store Front, 9’0” x9’0”; (2) Partition mounted doors; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down carpet; 2 x 4 Acoustical ceiling 8’9” AFF, painted walls; AC, Fire Sprinkler, Electrical 2 x 4 lights and standard electrical receptacles.  Office Furniture

Room number 3} Storefront door and glazing 9’0” x 9’0”, (1) Partition mounted door; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down carpet; 2 x 4 Acoustical ceiling 8’9” AFF, painted walls; AC, Fire Sprinkler, Electrical 2 x 4 lights and standard electrical receptacles. Office Furniture

Room number 4} (3) rear entrances doors, (2) 3’0” x 6’8” doors and wood frames; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF; painted on gray epoxy floor over past glue, (poor shape); plastic laminated base and counter top; 2 x 4 Acoustical ceiling 8’4” AFF, painted walls; AC, Fire Sprinkler, Electrical Service, 2 x 4 lights and standard electrical receptacles.

Room number 5} (1) partition mounted Door;  Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down VCT; 2 x 4 Acoustical ceiling 8’0” AFF, painted walls; AC, Fire Sprinkler, Electrical 2 x 4 lights and standard electrical receptacles, Phone board.

Room number 6} (1) partition mounted Door; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down VCT; plastic laminated reception counter and base, plastic laminated base and counter top; 2 x 4 Acoustical ceiling 8’0” AFF, painted walls; Toilet and Sink, AC, Fire Sprinkler, Electrical (1) 2 x 4 lights and standard electrical receptacles.

Room number 7} (1) partition mounted Door; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down VCT; 2 x 4 Acoustical ceiling 8’0” AFF, painted walls; Plastic laminated Toilet Partitions (2) Toilet and (1) Sink, AC, Fire Sprinkler, Electrical (2) 2 x 2 lights and standard electrical receptacles.

Room number 8} Open Breezeway; Stucco on block to 10’1” AFF , Bearing walls to deck,  bar joist bearing approx 12’0” AFF, Floor Stamped concrete; Plaster stucco ceiling 10’1” AFF suspended from bar joist, painted walls; No AC, No Fire Sprinkler, Electrical (a part of shopping plaza House Service)  lights and no receptacles.

Room number 9} (1) Door entrance in block wall;  Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down carpet; 2 x 4 Acoustical ceiling 9’0”AFF, painted walls; AC, Fire Sprinkler, Electrical Service (a part of shopping plaza House Service)  2 x 4 lights and standard electrical receptacles.

Room number 9A} (1) Door partition mounted;  Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down carpet; 2 x 4 Acoustical ceiling 9’0”AFF, painted walls; AC, Fire Sprinkler, Electrical Service (a part of shopping plaza House Service)  2 x 4 lights and standard electrical receptacles.

Room number 9B} (1) Door partition mounted;  Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down carpet; 2 x 4 Acoustical ceiling 9’0”AFF, painted walls; AC, Fire Sprinkler, Electrical Service (a part of shopping plaza House Service)  2 x 4 lights and standard electrical receptacles.

Room number 10} Open Breezeway; Stucco on block to 10’1” AFF , Bearing walls to deck,  bar joist bearing approx 12’0” AFF, Floor Stamped concrete; Plaster stucco ceiling 10’1” AFF suspended from bar joist, painted walls; No AC, No Fire Sprinkler, Electrical (a part of shopping plaza House Service)  lights and no receptacles.

Room number 11} Open Breezeway; Stucco on block to 10’1” AFF , Bearing walls to deck,  bar joist bearing approx 12’0” AFF, Floor Stamped concrete; Plaster stucco ceiling 10’1” AFF suspended from bar joist, painted walls; No AC, No Fire Sprinkler, Electrical (a part of shopping plaza House Service)  lights and no receptacles.

Room number 12} (1) Door entrance in block wall; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down vct; 2 x 4 Acoustical ceiling 8’0 AFF, painted walls; AC, Fire Sprinkler, Electrical (a part of shopping plaza House Service)  2 x 4 lights and standard electrical.

Room number 12A} (1) partition mounted Door; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down VCT; 2 x 4 Acoustical ceiling 8’0” AFF, painted walls; FRP panels to 8’0”; Plastic laminated Toilet Partitions (2) Toilet and (2) lav’s in Plastic Laminated Counter top; AC, Fire Sprinkler, Electrical (a part of shopping plaza House Service)  (1) 2 x 2 lights and standard electrical receptacles.

Room number 13} (1) Door partition mounted; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down vct; 2 x 4 Acoustical ceiling 10’0”  AFF, painted walls; AC, Fire Sprinkler, Electrical (a part of shopping plaza House Service)2 x 4 lights and standard electrical.

Room number 14} (1) Door entrance in block wall; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down vct; 2 x 4 Acoustical ceiling 8’0 AFF, painted walls; AC, Fire Sprinkler, Electrical (a part of shopping plaza House Service)  2 x 4 lights and standard electrical.

Room number 14A} (1) partition mounted Door; Drywall partitions, framed to bottom of bar joist approx 10’0” AFF, Drywall hung to 10’0” AFF;  glue down VCT; 2 x 4 Acoustical ceiling 8’0” AFF, painted walls; FRP panels to 8’0”; Plastic laminated Toilet Partitions (2) Toilet, (2 Urinals, and (2) lav’s in Plastic Laminated Counter top; AC, Fire Sprinkler, Electrical (a part of shopping plaza House Service)  (3) 2 x 2 lights and standard electrical receptacles.

Base Bid

Alternates # 1

Remove approximately 58 LF x 11’0” of concrete masonry bearing wall separating room 8 and 10 from room 9.

Remove wall and slab in (4) 3’ locations sawcut and remove slab in (4) 3’0” x 3’0”  locations space locations approx 19.34 LF apart. Build new foundation slab 12” thick,  reinforce with (4) # 5 at 6” oc EWTB also install 4 anchor bolts per each location Furnish ½” x 12” by combined welded length 58 LF each side beam plates thru bolted with threaded roods washers and nuts thru 8” x 8” tie beam in a stitch pattern, Furnish & install (4)  4” x 4” x 3/8” steel columns with top and bottom plates approximately 12’0” long with anchor bolts for base and weld and bolt top plates. Then remove balance of wall approx 46 LF x 11’0”. Repair floor where wall was removed and columns installed. Floor should be flat from one surface to another.

Door changes, Pair of HM Doors and Frame in room 4 will be required to change to 45 Min. rating, Door leading into  room 11 from expanded 9 will be required as a 45Min rated Door and frame. Doors and Frames for rooms 12 and 14 adjacent to expanded room 9 will have to be upgraded to 45 min. rated openings.

HVAC

Modified electrical fixtures from 3 lamp in rooms 8 and 10 to 4 lamp.

Demolition

- Room 4

- Cap plumbing in short wall, remove drinking fountain

- safe off electric in short wall,

- remove drywall and partition framing for short wall,

- remove drywall framing and drywall for new door opening into room 8 approximately 8’0” 7’2”

- Sawcut masonry opening for new 6’0” x 7’0” pair of doors approx 7.68 x 7.83

- Room 6

- Saw cut concrete floor 2’0’ x 2’0” for capping plumbing for toilet.

- Cap plumbing in wall and below slab, remove Toilet and sink.

- Tie up Duct and AC grill in ceiling for reinstall

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- remove drywall and partition framing for 3 partition walls

- Remove Door and Frame, save for reuse.

- Remove acoustical ceiling

- Remove VCT Floor tile, remove glue off of floor.

- Room 7

- Saw cut concrete floor  (2) 2’0’ x 2’0” for capping plumbing for toilet.

- Cap plumbing in wall and below slab, remove (2) Toilet and (1) sink.

- Tie up Duct and AC grill in ceiling for reinstall

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- remove drywall and partition framing for 2 partition walls

- Remove Door and Frame, save for reuse.

- Remove acoustical ceiling

- Remove VCT Floor tile, remove glue off of floor.

- Room 8

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- Remove suspended plaster ceiling.

- Room 9

- Tie up Duct and AC grill in ceiling for reinstall

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- remove drywall and partition framing from block walls

- Remove acoustical ceiling

- Remove Glue down carpet, remove glue off of floor.

- Room 9A

- Tie up Duct and AC grill in ceiling for reinstall

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- remove drywall and partition framing for all walls  partition and block walls.

- Remove window in masory wall approx 4’0 x 4’0” where door is located below

- sawcut masonry opening for new 8’0” x 8’0” pair of doors approx =97.68 x 8.83

- Remove Door and Frame, save for reuse.

- Remove acoustical ceiling

- Remove Glue down carpet remove glue off of floor.

- Room 9B

- Saw cut concrete floor  (2) 2’0’ x 2’0” for capping plumbing for wash basin

- Cap plumbing in wall and below slab, (1) wash sink.

- Tie up Duct and AC grill in ceiling for reinstall

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- remove drywall and partition framing for all walls  partition and block walls

- Remove Door and Frame, save for reuse.

- Remove acoustical ceiling

- Remove VCT Floor tile, remove glue off of floor.

- Room 10

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- Remove suspended plaster ceiling.

- Room 11

- safe off electrical  in wall and tie up lighting fixture above ceiling,

- Remove suspended plaster ceiling.

- Room 12  No Work

- Room 12A

- Remove Toilet and replace with toilet removed from room 6 or 7.

- Repair (1) faucets at sinks

- Remove Plastic Laminate partition, save for modification and reuse

- Room 13  No Work

- Room 14  No Work.

- Room 14A

- Re work Flush Valve at (1) Urinal, furnish new flush valve at Urinal # 2.

- Repair (2) faucets at sinks

Concrete & Masonry

- Room 4

- Formwork opening for new 6’4” x 7’2” for pair of doors pre cut approx 7.68 x 7.83

- Room 6

- Pour back (1) concrete floor repair 2’0’ x 2’0” for capping plumbing for toilet.

- Room 7

- Pour back (2) concrete floor repair 2’0’ x 2’0” for capping plumbing for toilet.

- Room 8

- Construct Concrete and masonry wall 8’0” x 12’0” to enclose breezeway,

- dowel into existing masonry wall on either side 16” oc and at slab dowel rods 16” oc

- Sawcut and install Volclay waterstop in concrete slab, invert 1st course beam block and pour solid (Similar to line 86 in scope of work)

- Furnish and install balance of wall, complete top of wall with 8” x 16” tie beam with (4) # 5 doweled into existing beams

- Room 9A

- Formwork and concrete for opening for new 8’2”” x 8’2” pair of doors pre cut approx =97.68 x 8.83

- Room 9B

- Pour back (1) concrete floor repair 2’0’ x 2’0” for capping plumbing for wash sink.

- Room 10

- Construct Concrete and masonry wall 4”10” x 9’8” to enclose breezeway,

- dowel into existing masonry wall on either side 16” oc and at slab dowel rods 16” oc

- Sawcut salb 3/8” x ¾”and install Volclay RX Type 102 waterstop on concrete slab, invert 1st course beam block and pour solid

- Furnish and install balance of wall, complete top of wall with 8” x 16” tie beam with (4) # 5 doweled into existing beams

- Room 11

- Construct Concrete and masonry wall 8’0” x 12’0” to enclose breezeway,

- dowel into existing masonry wall on either side 16” oc and at slab dowel rods 16” oc

- Sawcut and install Volclay waterstop concrete slab, invert 1st course beam block and pour solid (Similar to line 86 in scope of work)

- Furnish and install balance of wall, complete top of wall with 8” x 16” tie beam with (4) # 5 doweled into existing beams.

- Alternate Construct opening in Block wall for 3’0” x 7’0” HM Door and Frame Accessing room number 3 from room # 11

Doors Frames and Hardware

- Room 4

- Supply and install (1) pair 3’0” x 7’0” solid core birch doors with HM frame with building standard hardware

- Room 9

- Furnish and install  (1) 3’0” x 7’0” 16ga. HM Frame and 18 ga.HM Door with standard tested hardware meeting building code for 45 Min Fire Rating

- Room 10

- Supply and install (1) 3’0” x 7’0” 16ga. HM Frame and 18 ga.HM Door with standard tested hardware meeting building code for wind and impact requirements as a tested assembly

- Room 11

- Supply and install (1) pre-hung birch doors and  frame with building standard hardware.

- Alternate Supply 3’0 x 7’0” 18 ga. HM Frame and 3’0” x 7’0” Solid PCW birch door with standard privacy lock RH swing into room 3

Overhead Door

- Room # 9A Furnish and install 8’0” x 8’0” rollup door, Door to be current code rated min. 140 mph wind and  impact rated. Manually operated.

Stucco

- Rooms # 8, 9, 10, and 11 where exterior walls have been added or modified install a 3 coat stucco textured finish to closely match existing wall color and texture

Drywall.

- Room # 4

- Furnish framing and drywall to repair to finished opening for a pair of 3’0” x 7’0” opening

- Room # 6

- Skim existing wall to repair where walls from room 6 were removed

- Room 7

- Skim existing wall to repair where walls from room 7 were removed.\

- Rooms 8,9, and 10

- Exterior walls furnish Rigid R-11 insulation stick pinned to block walls from floor to deck

- Room 11.

- Furnish and install 25 Metal studs 16” oc w / 5/8” type x drywall each side finished to a level 4 finish

- Install R-11 insulation in wall cavity finish w/  taped and mudded at deck w 3m Fire caulk

- Rooms 12, 12A, 13, 14 and 14A  where the abut rooms 10 and 11 continue vertical walls above ceilings to deck  treat as a smoke wall

Acoustical Ceiling

- Room # 1 reinstall 6 new ceiling tiles in rooms 1 and 2 to match room where existing wall once stood.

- Room # 4 reinstall new ceilings in rooms 6 and 7 to match room existing height in room 4.

- Room # 11 Optional Cost Furnish and install new ceiling in room 11, Ceiling 2 x 4 layin 8’9” AFF .

Flooring

- Room 1 Repair carpet where wall relocated from, use remnant and closely match.

- Room 2 Repair carpet where wall removed, use remnant and closely match.

- Rooms 1, 2, and 3. Optional cost Remove all carpet and replace with VCT tile.

- Room # 4 install epoxy floor in rooms 6 and 7

- Room 11 install floor flattener over stamped concrete

- Room 11 Install VCT tile and base

Painting

- Room # 1, repair where wall was relocated from and to

- Room # 2, Repair where wall was relocated to and remove from

- Room # 3, Repair where wall was removed from.

- Room #3 Alternate, repair wall where Door installed from room 11 into room 3

- Room # 3Alternate paint 2 coats on Primed HM Frame and Prime and paint 2 coats on 3;’0” x 7’0” wood door

- Room # 4 repair wall where rooms 6 and 7 once attached and between

- Room # 4 paint 2 coats on pre primed HM Frame and paint prime and 2 coats on new pair of 3’0” x 7’0” SCW doors.

- Rooms # 8, 9, 10, and 11 where exterior walls have been added or modified and stucco finish applied prime and paint new and existing openings to closely match existing wall color and texture.

- Room #s 8,9,9a,9b,10,11

- Room # 8 &10 Optional cost to furnish and install Sonneborne 700 B dampproofing sealer on newly enclosed block wall from grade to 12’1” AFF.

- Room # 9 Optional cost to furnish and install Sonneborne 700 B dampproofing sealer on newly exposed block wall from grade to 12’1” AFF including former rooms 9A and 9B.

- Room 10 paint 2 coats on Primed HM Frame and Prime and paint 2 coats on 3;’0” x 7’0” SCW door

- Rooms 8,9,10 and 11 Optional cost Fog structural steel and barjoist including deck except for  mechanical ductwork.

- Room 12,12A, 13, 14, and 14A No Work

Bath Accessories

- Room 12A

- Furnish and install a partition wall between two toilets.

- Furnish and install (4) 42” long Grab rails.

- F & I (2) Toilet Paper dispensers

- Optional cost if 2 stall doesn’t meet ADA configure stall with 1 toilet

- Room 14A

- Furnish and install a partition wall between two toilets.

- Furnish and install (4) 42” long Grab rails.

- F & I (2) Toilet Paper dispensers.

- Optional cost if 2 stall doesn’t meet ADA configure stall with 1 toilet

Plumbing

- Room # 9  Install free standing laundry sink where formerly existed a mop sink in corner of room 9B

- Room 12A Repair faucet in lavatory, replace toilet with one of toilets taken from rooms 6 or 7

- Room 14A Repair both faucets in counter mounted lavatory, add flush valve to one urinal and repair the other urinal. Confirm both toilets operational

- Room 12A and 14A Optional price remove and cap one of 2 existing toilets in each room

HVAC

- Room 4 install grills as necessary due to removal of walls and ceilings in rooms 6 and 7

- Room 8, 10,11,12,12A,13,14,14A  Furnish and install New 3 ton split system fiber glass and flex ductwork as required for new layout.

- Room # 9 Relocate existing 3.5 ton split unit air handler within ceiling of room 9 furnish  new supply and return grills as required. 
alternate Furnish new 3.5 ton split unit with air filtration capabilities for dust control

- Furnish and install 2 new Remote test stations with audible and visual alert matched to existing duct smoke detectors.

- Check-out and service existing equipment.

- Furnish independent Comfort test and balance.

Fire Sprinkler

- Room number 4 Verify location of sprinkler heads and adjust height due to modified ceiling height in former rooms 6 and 7

- Room numbers 8, 10, and 11 furnish sprinkler heads in these room turn heads up as no ceilings are included in base bid.

- Room # 9, 9a and 9b Turn all sprinkler heads in an uprite position all ceiling to be removed

- Modify existing sprinkler heads to accommodate new tenant build out. Include engineering and drawings for permit. Include cost for sprinkler permit fee

Electric

- Rooms 8,9,10,11,12,12A, 13, 14, 14A. Furnish and install new service. Locate service in meter room shown abutting room 14A.

- Rooms 8,9,10,11,12,12A, 13, 14, 14A. Disengage existing rooms from House panel electric while enabling all downstream to continue to function. House panel located in meter room shown abutting room 14A.

- Furnish and install roof mounted disconnect for a 3 ton split unit.

- Relocate existing 3.5 ton split unit air handler within ceiling of room 9.

- Furnish alternate new 3.5 ton split unit with air filtration capabilities for dust control

- Room 2 relocate (2) 2 x 4 lay in light fixtures displaced by wall relocation

- Room 1 and 2 Furnish 2 duplex receptacles on each side of new wall

- Room number 4 Reinstall light fixtures tied into ceilings due to removal of rooms 6 and 7.

- Rooms 8, 10, and 11 furnish suspended (8) 2 x 4 fixtures 3 T 5 lamps suspended by chains from structure above. Lighting to be hung at 9’0” AFF

- Room 11 Optional price if Acoustical ceiling is installed (2) 2 x 4 will become (3) 2’0” x 2’0” 3 lamp T5 fixtures

- Room # 9 Furnish and suspend (12) 2 x 4 fixtures 4 T-5 lamps  suspended by chains from structure above. Lighting to be hung at 10’0” above finish floor

- Rooms 8,8,10,11 Please note all walls are block, therefore all wall electric will surface mounted conduit and boxes.

- Room number 9 will have 10 pairs of duplex receptacles all other rooms to have receptacles as required by code

- Provide and install exit signs and emergency lighting per code.

- Install dedicated outlets, general purpose outlets, and low voltage stubs.

- Furnish a allowance for Fire alarm system to tie in with existing system.

- Furnish cost for Shunt trip for tenant with multiple services located in different areas