BUilding Technology Svcs, inc. license# cac058685

13833 WELLINGTON TRACE, E4-157 WELLINGTON, FL. 33414 561-712-1126 fax 561-712-1407

June 4, 2010

Mick Satur

Seacoast National Bank

RE: LIQUID LINE FILTER/DRIER REPLACEMENT @ 3001 PGA BOULEVARD

We hereby submit specifications and estimate to:

Remove and replace (6) liquid line driers.  Pump freon (Refrigerant-22) out of system, open system to atmospheric pressure, sweat existing drier off of liquid line with acetylene/oxygen torch, braze new drier onto liquid line with 15% silver brazing rod, coat ends of new drier with cold galvanizing compound, use vacuum pump to bring system into a full vacuum, release freon charge back into system, re-start equipment and add freon as necessary to adjust charge to full. This final step is necessary as a small amount of freon is usually lost during the first step of the process.

Price………………………$1,680.32 ($280.05 each)

Maintenance Discount…….-15%

Quantity Discount…………-5%

Final Price Each………….$1,344.26 ($224.04 each)

Price to include: Sales tax, labor and miscellaneous materials.

Approved by: ___________________________       _______

Seacoast National Bank 		     Date

Thank you,

Cindy Campbell,

B.T.S., Inc.